## AES Register Bank

The Register Bank contains 13 registers, each 32 bits wide. The registers are organized into four groups:

- Control and Status Register (CSR)

- Key Registers

- Text Input Registers

- Text Output Registers

| Offset | Register | Width | Access | Purpose |
| --- | --- | --- | --- | --- |
| 0x00 | CSR | 32 bits | R/W | Controls the AES operation & indicates completion status |
| 0x04 | KEY0 | 32 bits | R/W | Stores Key [31:0] |
| 0x08 | KEY1 | 32 bits | R/W | Stores Key [63:32] |
| 0x0C | KEY2 | 32 bits | R/W | Stores Key [95:64] |
| 0x10 | KEY3 | 32 bits | R/W | Stores Key [127:96] |
| 0x14 | TEXT_IN0 | 32 bits | R/W | Stores Input [31:0] |
| 0x18 | TEXT_IN1 | 32 bits | R/W | Stores Input [63:32] |
| 0x1C | TEXT_IN2 | 32 bits | R/W | Stores Input [95:64] |
| 0x20 | TEXT_IN3 | 32 bits | R/W | Stores Input [127:96] |
| 0x24 | TEXT_OUT0 | 32 bits | R | Provides Output [31:0] |
| 0x28 | TEXT_OUT1 | 32 bits | R | Provides Output [63:32] |
| 0x2C | TEXT_OUT2 | 32 bits | R | Provides Output [95:64] |
| 0x30 | TEXT_OUT3 | 32 bits | R | Provides Output [127:96] |

# CSR — Control and Status Register for Normal Cipher

Address: 0x00
Width: 32 bits
Access: Read/Write

Only two bits are currently defined:

| Bit | Name | Access | Description |
| --- | --- | --- | --- |
| 31:17 | Reserved | — | Reserved for future use; reads as 0 |
| 16 | DONE | R | Indicates that the AES operation has completed |
| 15:1 | Reserved | — | Reserved for future use; reads as 0 |
| 0 | LOAD | R/W | Used to initiate/load the AES operation |

### LOAD — CSR[0]

The LOAD bit is controlled by software.

When the processor writes a 1 to CSR[0], the Register Bank asserts the corresponding AES load control signal. This indicates that the AES core should begin processing the currently loaded key and input data.

The processor can therefore use this bit to control when the AES operation is initiated.

### DONE — CSR[16]

The DONE bit is a hardware-generated status bit.

It indicates that the AES core has completed its operation and that the output data is available in the TEXT_OUT registers.

The software can read CSR[16] to determine whether the AES operation has completed.

Important: DONE is not intended to be written by software. Its value is driven by the AES core's completion signal.

# CSR — Control and Status Register for Inverse Cipher

Address: 0x00
Width: 32 bits
Access: Read/Write

Only four bits are currently defined:

| Bit | Field | Access | Description |
| --- | --- | --- | --- |
| 31:18 | Reserved | — | Reserved for future use; reads as 0 |
| 17 | KEY_DONE | R | Indicates that the key loading/key expansion operation is complete |
| 16 | DONE | R | Indicates that the Inverse Cipher operation is complete |
| 15:2 | Reserved | — | Reserved for future use; reads as 0 |
| 1 | KEY_LOAD | R/W | Requests/initiates key loading for the Inverse Cipher |
| 0 | LOAD | R/W | Requests/initiates the Inverse Cipher operation |

# KEY Registers

The AES-128 key is 128 bits wide, while each Register Bank register is 32 bits wide.

Therefore, the complete AES key is divided across four 32-bit registers.

| Register | Address | Bits of AES Key | Access |
| --- | --- | --- | --- |
| KEY0 | 0x04 | [31:0] | R/W |
| KEY1 | 0x08 | [63:32] | R/W |
| KEY2 | 0x0C | [95:64] | R/W |
| KEY3 | 0x10 | [127:96] | R/W |

### KEY0

Address: 0x04
Width: 32 bits
Access: Read/Write

KEY0 stores the least-significant 32 bits of the 128-bit AES key.

It corresponds to: AES Key [31:0]

### KEY1

Address: 0x08
Width: 32 bits
Access: Read/Write

KEY1 stores the next 32 bits of the AES key.

It corresponds to: AES Key [63:32]

### KEY2

Address: 0x0C
Width: 32 bits
Access: Read/Write

KEY2 stores the next 32 bits of the AES key.

It corresponds to: AES Key [95:64]

### KEY3

Address: 0x10
Width: 32 bits
Access: Read/Write

KEY3 stores the most significant 32 bits of the AES key.

It corresponds to: AES Key [127:96]

The four registers together form the complete 128-bit AES key:

Thus, the Register Bank converts four 32-bit software-visible registers into the 128-bit key input required by the AES core.

# TEXT_IN Registers

The AES plaintext/input is also 128 bits wide. It is therefore divided into four 32-bit registers.

| Register | Address | Bits of Input | Access |
| --- | --- | --- | --- |
| TEXT_IN0 | 0x14 | [31:0] | R/W |
| TEXT_IN1 | 0x18 | [63:32] | R/W |
| TEXT_IN2 | 0x1C | [95:64] | R/W |
| TEXT_IN3 | 0x20 | [127:96] | R/W |

### TEXT_IN0

Address: 0x14
Stores input data bits [31:0].

### TEXT_IN1

Address: 0x18
Stores input data bits [63:32].

### TEXT_IN2

Address: 0x1C
Stores input data bits [95:64].

### TEXT_IN3

Address: 0x20
Stores input data bits [127:96].

Together, they form the complete 128-bit AES input.

The processor writes the plaintext into these four registers before initiating the AES operation.

# TEXT_OUT Registers

The AES core produces a 128-bit output. This output is divided into four 32-bit registers so that the processor can read it through the Register Bank.

| Register | Address | Bits of Output | Access |
| --- | --- | --- | --- |
| TEXT_OUT0 | 0x24 | [31:0] | R |
| TEXT_OUT1 | 0x28 | [63:32] | R |
| TEXT_OUT2 | 0x2C | [95:64] | R |
| TEXT_OUT3 | 0x30 | [127:96] | R |

### TEXT_OUT0

Address: 0x24
Provides output bits [31:0].

### TEXT_OUT1

Address: 0x28
Provides output bits [63:32].

### TEXT_OUT2

Address: 0x2C
Provides output bits [95:64].

### TEXT_OUT3

Address: 0x30
Provides output bits [127:96].

Together, they reconstruct the complete AES output.

These registers are read-only because the output is generated by the AES hardware rather than written by the processor.